BIPS Description:

General Conditions:

3.xxx	Strapping / Fixation:

Strapping shall be installed at intervals of every 0.5 m and at all joints.

The material must be nylon or glass-fiber-reinforced nylon, corrosion-resistant, dimensionally stable, and non-condensing.

Bygningsdelsbeskrivelse:

4.xxxx	Strapping / Fixation:

Strapping shall be installed at intervals of every 0.5 m and at all joints/couplings and changes of direction.

The system must be directionally stable and able to withstand both compressive and tensile forces (uplift).

During installation, backfilling with raw soil is not permitted; uncompacted pipe gravel must be used. For embedment depths greater than 25 cm below insulation, lightweight materials such as expanded clay (LECA) pellets or polystyrene (EPS) shall be used. After the floor insulation has been installed, the strapping shall be cut off 4–5 cm above the insulation prior to placing reinforcement mesh in the floor structure (minimum embedment depth of 4 cm in in-situ concrete). The system must not be subjected to lateral loads during installation. If the system is subjected to improper loading or if the strapping must be repositioned, it shall be discarded and replaced with a new unit. (The system must not be disassembled and reused).